# Carboni 1 Minicomputer

Technical Reference

Jules Carboni, 2016-2024	First Edition, 17 November 20XX

## Contents

Part One — Introduction	Page 3

Background, For Beginners, System Specifications

Part Two — Design Details	Page 5

Organisation, Clock, Arithmetic Logic Unit, Registers, Memory, External Storage, Instruction Decoding & Control Logic, Input & Output

Part Three — Operation Details	Page X

Power Supply, Front Panel Switches & Indicators, Settings & Modes of Operation, Loading a Program, Using the Memory, Memory Addressing, Operating Hints

Part Four — Instruction Set	Page X

Control Pins, Command Instructions, Data Transfer Instructions, Accumulator Instructions, Branching Instructions

Part Five — Programming Guide	Page X

Fundamentals of Programming, A Simple Program, Assembly Language, Machine Code, Debugging, Sample Programs, Concept of Operating Systems

Appendix One — Maintenance Information	Page X

Hardware Vulnerabilities, Maintenance Advice, Bad Circuitry, Bad Microcode

Appendix Two — Technical Information	Page X

Instruction Reference Card, Parts List, System Schematics, Data Sheets

Appendix Three — Supplementary Information	Page X

Boolean Algebra, Electronic Logic, Number Systems & Binary, Storing Negative Numbers

## Part 1 — Introduction

Background	Page 4

Rationale, Design

For Beginners	Page 4

Required Understanding

System Specifications	Page 4

Overview

### 1.1 Background

The Abacus 1 was created as a research project with the purpose of exploring how computers function at the lowest level.

Abacus 2 uses an evolution of the Abacus 1 architecture (based on Albert Paul Malvino’s SAP-1/2/3 architecture). The computer is Turing complete; it can compute anything computable within the constraints of its memory size and processing speed (explained further in parts 2.5 & 2.2 respectively). The Abacus 2 instruction set architecture also provides integration with AMD’s AM9511 floating point unit.

The physical construction of Carbon 1 is highly reminiscent of home computers of the 1970s, i.e. MITS Altair 8800. The case opens to allow access to the internals. The “slot card” design allows components—called “modules”—to be easily removed for repair or replacement. The front panel uses LEDs to indicate the logical states of every component in the system, e.g. the registers, to assist the operator in debugging.

### 1.2 For Beginners

This operating manual will describe the architecture, functions and processes of the Abacus 2. An understanding of Boolean algebra, electronic logic, number systems (specifically the binary system) is assumed knowledge. It is highly recommended to understand these concepts fully before continuing. To assist beginners, the introductory section of MITS’ ‘Altair 8800 Operators Manual’ has been included in appendix three “Supplementary Information” of this manual. The excerpt details these concepts to a beginner audience in sufficient detail for comprehension of this manual.

### 1.3 System Specifications

This section of the manual outlines the key technical specifications of the Abacus 2 processor. For more detail see part two “Design Details”.

| Power | ATX standard |
| --- | --- |
| Architecture | 16-bit, CISC, Von Neumann |
| Instruction Set | Abacus ISA, 44 instructions |
| Clock | FM, digital control, ~1Hz–16MHz |
| ALU | 16-bit, 74181 based |
| Memory | 4KB–16MB, 8-bit bytes, SRAM |
| Bus | 16-bits, shared, internal/external |

## Part 2 — Design Details

Organisation	Page 6

Modules

Clock	Page 7

Astable & Monostable Modes, Speeds

Arithmetic Logic Unit	Page 7

Arithmetic and Logic Operations

Registers	Page 8

Importance of Accumulator, Other Registers, Flags Register

Memory	Page 9

Word Length, Addressing, Program Counter

External Storage	Page 9

Punched Tape Reader

Instruction Decoding & Control Logic	Page 9

Word Length, Control Lines, Fetch Phase

Input & Output	Page 9

Front Panel Interface, Seven-Segment Display, Internal State Indicators

### 2.1 Organisation

The organisation of the Abacus 1 is based around “modules”. Each major component of the system comprises its own module. Data is shared between modules on the system’s bus.

Clock

MAR

Memory

Inst. Register

Counter

Accumulator

ALU

Register B

Output

Tape

Inst. Decoder

Flags

B

u

s

12 LSB

12 LSB

4 MSB

12 LSB

12 LSB

4 LSB

Hlt

Co Ce

J Jc Jz

Mi

Ri Ro

Ii Io

Ai Ao

Bi

Fi Fo

Eo Su Xr

An Or Nt

Oi

Below can be observed the order of the Abacus 1’s modules as they are assembled.

Clock

Counter

MAR

Memory

Accumulator

ALU

Flags

Register B

Instruction

Output

Tape

TEMPORARY GRAPHIC

### 2.2 Clock

The clock synchronises the actions of the whole system, keeping every component coordinated. Each component of the system activates and operates in time with the pulsing of the clock—the more frequent the clock pulses the faster the system operates.

The Abacus 1 internal clock has an adjustable clock speed in the range of 0.721Hz to 4810Hz (~5KHz). The duty cycle ranges from 50% high at 0.7Hz to 66.67% high at 4.8KHz. The clock is not regulated by a crystal, using only a 555 timer integrated circuit.

Each fetch-decode-execute cycle requires two to five clock pulses to complete, resulting in an execution rate in the range of 962 to 2405 instructions per second (IPS), with an approximate weighted average of 1150 IPS.

The Hlt control feeds back to the clock and halts it, so that neither a-stable nor monostable modes will output a clock pulse to the system, effectively freezing the computer until it is reset. This control is tied to the Hlt instruction which can be used in programs.

### 2.3 Arithmetic Logic Unit

At the heart of every computer is the Arithmetic Logic Unit, a device for performing operations on numbers. The Abacus 1’s ALU can perform six functions:

- Arithmetical Addition

- Arithmetical Subtraction

- Logical Exclusive Or

- Logical And

- Logical Or

- Logical Not

Note; all arithmetic operations are performed by addition (subtraction is just inverse addition); this means it is easily possible to write programs to perform multiplication and division, despite these functions not being implemented in hardware. Shift and rotate functions can also be implemented this way.

After the ALU has finished its operation, it will update the Flags Register. This enables the ALU to be used to implement comparative instructions that are not available as explicit instructions in the instruction set, such as “greater than” and “equal to”.

### 2.4 Registers

Every register in the Abacus 1 minicomputer serves a special purpose, there are no “general purpose” registers that can be used freely for variable storage—variables must be stored in memory. The unique features and uses of each register will be described in the following parts.

The A Register, known as the accumulator, is imperative to the functionality of the Abacus 1. It holds the current value being operated on, particularly during arithmetic and logic operations, and is where the results of these operations are eventually loaded/located. The accumulator is accessible via instructions and can be read and written to directly.

The B Register is electrically identical to the accumulator and holds intermediate or secondary values during operations that require the arithmetic and logic unit. However, unlike the accumulator, the B Register is not accessible to the end user and cannot be read or written to directly.

The Instruction Register takes in the four most significant bits on the bus (the instruction) and relays the value to the instruction decoder/control logic component. This register cannot be accessed by the user at all.

The Memory Address Register will be discussed in 2.5 “Memory” of this manual.

The Program Counter will be discussed in 2.5 “Memory” of this manual.

The Flags Register operates differently to the other registers. It stores the status of four conditions which may or may not be affected by the result of an operation. It is only readable through the Ldf instruction and cannot be read or written to directly. The four status bits are:

- Carry bit—This bit is set to 1 if a carry has occurred during an arithmetical or logical operation. The bit is reset to 0 is no carry occurs.

- Zero bit—This bit is set to 1 if the result of arithmetic or logic instructions is zero and reset to 0 is the result is not zero.

- Sign bit—This bit is set to show the sign of a result. If set to 1, the result is negative; if set to 0 the result is positive. The sign bit reflects the condition of the most significant bit in the result (bit number 15).

- Overflow bit—This bit indicates that an operation incorrectly changed the sign bit of the result. This happens when the result of an arithmetic operation exceeds the maximum value that can be stored in the 16-bit ALU’s “sum” register.

The carry bit and zero bit stored in the Flags Register affect the results of instructions that rely on comparisons between numbers, such as conditional jump instructions. The sign bit and overflow bit have no effect on the program/system at all and are used only as indicators to the operator.

The bit widths of the registers in the Abacus 1 differ:

| Register | Bit Width |
| --- | --- |
| A | 16 bits |
| B | 16 bits |
| Instruction | 4 most significant bits |
| Memory Address | 12 least significant bits |
| Flags | 4 (least significant) bits |

Note; most/least significant bits (MSB/LSB) refers to the significant figures of a binary integer. For example, the most significant bit in 1000 is 1.

### 2.5 Memory

The Abacus 1 minicomputer contains four kilowords (4,096 words) of random access memory. Each word is 16-bits in length. When instructions are stored in memory, the four most significant bits constitute the instruction and the remaining twelve bits form the address (if necessary). Values can also be stored in memory using the full 16-bits of data per address.

The memory can be read and written to directly with the use of the front panel memory address and memory data switches.

The Memory Address Register takes in the twelve least significant bits on the bus (the address) and relays the value to the (random access) memory module. This register cannot be accessed by the user at all.

The Program Counter is a special 12-bit register which stores the address of the next program step (memory address) to be executed. The Program Counter is automatically advanced to the next sequential program address upon completion of the “fetch phase” of the instruction cycle. The PC is directly writable via jump and conditional jump instructions but cannot be read from.

### 2.6 External Storage

The Abacus 1 has a 16-bit system-wide bus. This bus extends beyond the internal components and can be accessed via 16 external pins. The Abacus 1 is fitted with an optional “tape reader” module that can automate the process of loading programs. A punched tape reader, or any other functionally compatible device, can be plugged into the 16 external bus pins for use with the Tape module.

### 2.7 Instruction Decoding & Control Logic

The Instruction Register (explained in part 2.4 “Registers”) is connected to the bus. When the fetch phase of the instruction cycle commences, the IR reads the contents supplied by the memory and splits it into two sections; the instruction (4 MSB) and the address (12 LSB). It forwards the address (12 LSB) to where it needs to go and increments the Program Counter. All the while, the Instruction Decoder has taken in the instruction (4 MBS) and fed the value into the three EEPROMs that comprise its control logic. The logic activates certain control signals at specific times in the instruction cycle, allowing many unique and complex instructions to be performed depending on the input value.

### 2.8 Input & Output

The Abacus 1 can be interfaced with the on-board switches or using a “tape compatible” input device. Its output consists of a five-digit seven-segment display (with one additional digit for signifying negative integers). The display can be hexadecimal or signed or unsigned decimal. Additionally, myriad status indicator LEDs report the contents of every internal register in the system.

## Part 3 — Operation Details

Power Supply	Page 6

temp

Front Panel Switches & Indicators	Page 7

Temp

Settings & Modes of Operation	Page 6

Clock Modes and Speed, Display Modes

Loading a Program	Page 7

temp

Using the Memory	Page 8

temp

Memory Addressing	Page 9

temp

Operating Hints	Page 9

temp

### 3.3 Settings & Modes of Operation

The clock has two operational modes; a-stable “pulse mode” and monostable “step mode”. Pulse mode is considered the normal operating mode, where the clock rhythmically pulses at its set speed. Step mode overrides the internal clock with a non-latching switch labelled Step on the front panel. When Step is pressed, the clock will go high until the switch is released. Note; a logical process is used when switching between modes to ensure the wire never floats.

The LED labelled Clock on the front panel indicates the computer’s current clock speed. The LED labelled Halt indicates whether the computer is halted… If so, the computer has finished executing the program in memory. When Halt is high, the clock will be stopped.

## Part 4 — Abacus 1 Instruction Set

Control Signals	Page 12

Types of Instructions, Respective Control Signals, Control Timing

Control Instructions	Page 13

Nop, Out, Hlt

Data Transfer Instructions	Page 13

Lda, Ldi, Sta, Ldf

Accumulator Instructions	Page 14

Add, Sub, Xor, And, Or, Not

Branching Instructions	Page 14

Jmp, Jc, Jz

### 4.1 Control Signals

The Abacus 1 has 15 basic machine language instructions grouped into the following four categories:

- Command Instructions

- Data Transfer Instructions

- Accumulator Instructions

- Branching Instructions

Each instruction is performed over a maximum of five clock pulses through a series of signals sent to many different components of the system. Listed below are all the modules of the Abacus 1 and their control signals.

| Module | Control Signals |
| --- | --- |
| Clock | Hlt |
| Program Counter | Ce, Co, J, Jc, Jz |
| Instruction Register | Ii, Io |
| Memory Address Register | Mi |
| (Random Access) Memory | Ri, Ro |
| Accumulator | Ai, Ao |
| Register B | Bi |
| Arithmetic Logic Unit | Eo, Su, Xr, An, Or, Nt |
| Flags Register | Fi, Fo |

The special control signal Tr (time reset) is activated after an instruction has finished executing if it does not use all five “time” periods. It asynchronously resets the control logic counter back to “time one”, beginning the fetch phase of the next instruction cycle.

Listed below are all the instructions of the Abacus 1 and their active controls for every “time”. The first two “times” comprise the “fetch” component of the instruction (fetch-decode-execute) cycle.

| Instruction | Active Controls |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | T1 | T2 | T3 | T4 | T5 |
| Nop | Mi, Co | Ro, Ii, Ce | Tr | – | – |
| Lda | Mi, Co | Ro, Ii, Ce | Io, Mi | Ro, Ai | Tr |
| Add | Mi, Co | Ro, Ii, Ce | Io, Mi | Ro, Bi | Eo, Ai, Fi |
| Sub | Mi, Co | Ro, Ii, Ce | Io, Mi | Ro, Bi | Eo, Ai, Su, Fi |
| Sta | Mi, Co | Ro, Ii, Ce | Io, Mi | Ao, Ri | Tr |
| Ldi | Mi, Co | Ro, Ii, Ce | Io, Ai | Tr | – |
| Jmp | Mi, Co | Ro, Ii, Ce | Io, J | Tr | – |
| Jc | Mi, Co | Ro, Ii, Ce | Io, Jc | Tr | – |
| Jz | Mi, Co | Ro, Ii, Ce | Io, Jz | Tr | – |
| Xor | Mi, Co | Ro, Ii, Ce | Io, Mi | Ro, Bi | Eo, Ai, Xr, Fi |
| And | Mi, Co | Ro, Ii, Ce | Io, Mi | Ro, Bi | Eo, Ai, An, Fi |
| Or | Mi, Co | Ro, Ii, Ce | Io, Mi | Ro, Bi | Eo, Ai, Or, Fi |
| Not | Mi, Co | Ro, Ii, Ce | Eo, Ai, Nt, Fi | Tr | – |
| Ldf | Mi, Co | Ro, Ii, Ce | Fo, Ai | Tr | – |
| Out | Mi, Co | Ro, Ii, Ce | Ao, Oi | Tr | – |
| Hlt | Mi, Co | Ro, Ii, Ce | Hlt | Tr | – |

The physical paths and connections of the control signals can be observed in part 2.1 “Organisation” of this manual.

### 4.2 Control Instructions

Nop – no operation. Syntax: 0000. This instruction carries out the “fetch phase” of the instruction (fetch-decode-execute) cycle but nothing more. It simply increments the program counter, progressing to the next memory location.

Out – output accumulator. Syntax: 1110. This instruction takes the current value loaded in the accumulator and pushed it to the display module. The value will be displayed on the five-digit seven-segment display.

Hlt – halt clock. Syntax: 1111. This instruction signals the clock to stop pulsing, freezing the computer at “time five”—the Hlt instruction’s “execute phase”.

### 4.3 Data Transfer Instructions

Lda – load accumulator. Syntax: 0001 <Address>. This instruction takes the value stored at the specified address in memory and loads it into the accumulator. It does not affect the contents in memory.

Ldi – load immediate. Syntax: 0101 <Integer>. This instruction takes the value specified (which would normally be an address) and loads it into the accumulator. It cannot load values less than zero or greater than 4095 (0x000-0xfff). This is because it only has 12-bits to work with. It does not access the memory at any time.

Sta – store accumulator. Syntax: 0100 <Address>. This instruction takes the value loaded into the accumulator and stores it at the specified address in memory. It will overwrite any memory contents at the specified address.

Ldf – load flags. Syntax: 1101. This instruction takes the value stored at the special flags register and loads it into the accumulator. The contents can then be masked to make use of the system’s many flags. It does not affect the contents in memory.

### 4.4 Accumulator Instructions

Accumulator instructions will update the flags register. Note; they can cause overflow errors.

Add – arithmetical addition. Syntax: 0010 <Address>. This instruction takes the value stored at the specified address in memory and adds it to the current value loaded in the accumulator.

Sub – arithmetical subtraction. Syntax: 0011 <Address>. This instruction takes the value stored at the specified address in memory and subtracts it from the current value loaded in the accumulator. It works by adding the twos-compliment of a number.

Xor – logical exclusive or. Syntax: 1001 <Address>. This instruction takes the value stored at the specified address in memory and “exclusively ors” it with the current value loaded in the accumulator.

And – logical and. Syntax: 1010 <Address>. This instruction takes the value stored at the specified address in memory and “ands” it with the current value loaded in the accumulator.

Or – logical or. Syntax: 1011 <Address>. This instruction takes the value stored at the specified address in memory and “ors” it with the current value loaded in the accumulator.

Not – logical not. Syntax: 1100. This instruction “nots” the current value loaded in the accumulator. It does not require any additional parameters.

### 4.5 Branching Instructions

Jmp – unconditional jump. Syntax: 0110 <Address>. This instruction will overwrite the program counter’s value with the value stored at the specified address in memory.

Jc – jump on carry. Syntax: 0111 <Address>. This instruction will overwrite the program counter’s value with the value stored at the specified address in memory only when the carry bit is set.

Jz – jump on zero. Syntax: 1000 <Address>. This instruction will overwrite the program counter’s value with the value stored at the specified address in memory only when the zero bit is set.

An instruction reference card is included in appendix three “Supplementary Information” of this manual.

## Part 5 — Programming Guide

Fundamentals of Programming	Page 16

Theory of Simulating Circuits, Methodical Approach to Programming

A Simple Program	Page 17

Defining a Problem, Creating a Flow Diagram

Assembly Language	Page 18

Purpose, Flow Diagram to Assembly Language, Representing Values

Machine Code	Page 19

Assembly Language to Machine Code, Hexadecimal Representation

Debugging	Page 20

Where Bugs Are Born, Tools to Use, Step-By-Step Process

Sample Programs	Page 21

Fibonacci Sequence, Multiplication, Loop Counter, Floor Division with Modulo

Concept of Operating Systems	Page 23

Purpose, Basic Application

### 5.1 Fundamentals of Programming

As has become apparent through the exploration of the Abacus 1 and its control logic, to be meaningfully applied, the computer must be programmed. Sets of digital instructions, called programs, routines, code or software, implement variations in hardware, altering the functionality of the computer. In programming a computer, the operator is effectively wiring an invisible circuit; and so long that the circuit does not change, the process which it carries out will not change. For example, imagine the programmed “circuit” is one that adds numbers; given this “circuit” or process remains the same, the outputs will always be the same for given inputs. Computers are a piece of variable hardware, that can be programmed to imitate a constant piece of hardware reliably.

The software instructions for the Abacus 1 must be inputted into the machine in the form of sequential 16-bit words known as machine language. Machine code will be discussed in part 5.3.

The basics of computer programming are quite simple. In fact, often the most difficult part of programming is defining the problem you with to solve with the computer. Below are listed the three main steps in generating a program.

- Defining the problem.

- Establishing an approach.

- Writing the program.

Once the problem has been defined, an approach to its solution can be developed. This step is simplified by making a diagram which shows the orderly, step-by-step solution of the problem. Such a diagram is called a flow diagram. After a flow diagram has been made, the various steps can be translated into the computer's language. This is the easiest of the three steps since all you need is a general understanding of the instructions and a list showing each instruction and its machine language equivalent.

The Abacus 1 has a small instruction set with extensive capability. For example, a program can cause data to be transferred between the computer's memory and the registers. The program can even cause the computer to make logical decisions. For example, if a specified condition is met, the computer can jump from one place in the program to any other place and continue program execution at the new place.

The Abacus 1 instruction set was described in detail in part four of this manual.

### 5.2 A Simple Program

Assume you wish to use the Abacus 1 to add two numbers located at two different memory locations and store the result elsewhere in the memory. Of course, this is a very simple problem, but it can be used to illustrate several basic programming techniques. Here are the steps used in generating a program to solve this problem:

- Define the problem—Add two numbers located in memory and store the result elsewhere in memory, then display the number and halt execution.

- Establish an approach—A flow diagram can now be generated:

Add number from second memory location.

Retrieve number from first memory location.

Store result in a new memory location.

Output result and halt the computer.

- Write the program—This step will be covered in the next part, part 5.3, after an explanation of assembly language.

Note; many flow diagrams can be created to compare the structure of different approaches to a problem.

### 5.3 Assembly Language

The software for any computer must be entered the machine in the form of binary words called machine language/code. Machine language programs are generally written with the help of mnemonics which correspond to the bit patterns for various instructions. For example, 0010 is an add instruction for the Abacus 1 and the corresponding mnemonic is Add. Obviously, the mnemonic Add is much more convenient to remember than its corresponding machine language bit pattern.

Ultimately, however, the machine language bit pattern for each instruction must be entered into the computer one step at a time.

We are now ready to translate the simple program from part 5.2 into assembly language. Translating the flow diagram into a language or format suitable for use by the computer may seem complicated at first. However, a general knowledge of the computer's organization and operation makes the job simple. In this case, the four-part flow diagram translates into five separate instructions:

| Line | Instruction | Address |
| --- | --- | --- |
| 0 | Lda (load value into accumulator) | 5 |
| 1 | Add (add value to accumulator) | 6 |
| 2 | Sta (store accumulator to memory) | 7 |
| 3 | Out (output result to display) | null |
| 4 | Hlt (halt/stop the computer) | null |

Make note of the line numbering, the program begins at line zero. This is because each line of code represents an address in physical memory, which is indexed from zero.

As described in part four, many instructions have arguments that are required for the instruction to execute. For example, the Lda instruction on line zero looks up the value at memory address three and moves it into the accumulator. If the address space were to be empty, the instruction would attempt to load memory address zero into the accumulator, which happens to be itself—a value of 0001—not what we intended.

For this reason, we need to store our values/integers in the memory too. The following lines of this program could look like this.

| Line | Instruction | Address |
| --- | --- | --- |
| 5 | 137 (the first number) 6200 (the second number) null (the result will go here) |  |
| 6 |  |  |
| 7 |  |  |

The lines of a program that contain values should always be written in the final lines of the program, sequentially after the program is halted. Values stored in memory can take up the whole 16-bits of data at any address, so were these placed in the middle of a program, they could be interpreted as an instruction and cause errors in execution. Shared instruction and data memory is one limitation of the Von Neumann architecture.

### 5.4 Machine Code

To put it simply, “machine code” is ones and zeroes. The mnemonics of assembly language are directly interchangeable with the raw binary instructions of machine code. This makes “assembling programs” simple.

Assembled, the program from part 5.3 would be written like this:

| Line | Instruction | Address |
| --- | --- | --- |
| … 0000 | 0001 | 0000 0000 0101 |
| … 0001 | 0010 | 0000 0000 0110 |
| … 0010 | 0100 | 0000 0000 0111 |
| … 0011 | 1110 | 0000 0000 0000 |
| … 0100 | 1111 | 0000 0000 0000 |
| … 0101 | 0000 | 0000 1000 1001 |
| … 0110 | 0001 | 1000 0011 1000 |
| … 0111 | 0000 | 0000 0000 0000 |

Make note of how binary representation of the decimal value 6200 flows into the instruction space.

The binary view of the program is not elegant nor space efficient. To neaten things up we can use hexadecimal notation to display this program; where in the contents the first digit represents the 4-bit instruction and the following three represent the 12-bit address.

| Memory Address | Memory Contents |
| --- | --- |
| 000 | 1 005 2 006 4 007 E 000 F 000 0 089 1 838 0 000 |
| 001 |  |
| 002 |  |
| 003 |  |
| 004 |  |
| 005 |  |
| 006 |  |
| 007 |  |

Congratulations, you’ve just written your first program for the Abacus 1 minicomputer! This code can be loaded into the memory of the computer and executed. Additionally, the code can be stored on paper or punch tape for storage outside of the volatile system memory for later recall/usage.

### 5.5 Debugging

Occasionally, the code loaded into the computer’s memory will not execute as expected. This is most likely the result of an error, called a “bug”, in the program. Rarely, this can be the result of a hardware failure; appendix one details the proper method of identifying and repairing faulty hardware. A bug often arises at three times in the process of programming a computer:

- When establishing an approach.

- When writing the program.

- When loading the program into memory.

There are a few things that will make debugging your code easier. These include:

- The program’s flow diagram.

- The program itself, written in assembly code.

- The instruction set reference, including all control signals (see part four).

The process of debugging can be both time consuming and infuriating; patience is essential. Rest assured, finally executing your program without error will be a very satisfying and rewarding experience. To successfully debug a program, follow these steps:

- Switch the clock mode from Pulse to Step; this ensures that the computer will not run before we’re ready.

- Load the program from scratch; sometimes a “buggy” program can alter its own code, further worsening the problem.

- Reset the computer by pressing the Reset switch. Initialisation of the computer is done.

- Look up the current instruction in your program and make note of the controls that it activates at each “time”. The clock should be at “time zero”, the first half of the “fetch phase”. It is safe to step the clock once, moving to “time one”, the second half of the “fetch phase”. If there are any erroneous controls at this point, there is an error in the computer’s microcode; see appendix 1.4 “Bad Microcode”.

- Step the clock by one, progressing to “time two”. Check that the instruction in the Instruction Register is the same as the one in your program. If not, there is an error in translation from assembly language to machine code.

- Compare the active controls to that of the instruction to check that there are no errors in the Microcode. If there are any erroneous controls at this point, there is an error in the computer’s microcode; see appendix 1.4 “Bad Microcode”.

- Repeat steps five to six until at “time four”.

- Compare the result of the instruction to the expected results described in your flow diagram and program (check register indicators). If there is a discrepancy, there may be an error in the usage of certain instructions, the implementation of code from the flow diagram, or simply a flow diagram that does not describe the process desired for execution.

- Repeat steps four to eight until a bug is found. If the program is not to blame for the erroneous operation of the computer, there may be an error in the wiring of the computer’s circuitry; see appendix 1.3 “Bad Circuitry”.

### 5.6 Sample Programs

Note; “–” denotes a variable value that is supplied by the programmer at the time of assembly.

Fibonacci Sequence:

| # | Assembly | Code | Comments |
| --- | --- | --- | --- |
|  | Top: |  |  |
| 0 | Ldi 0 | 7000 | Load 0 into accumulator (Acc). |
| 1 | Sta X | 400D | Store Acc value in X variable. |
| 2 | Out | E000 | Output Acc. |
| 3 | Ldi 1 | 5001 | Load 1 into Acc. |
|  | Next: |  |  |
| 4 | Sta Y | 400E | Store Acc in Y variable. |
| 5 | Out | E000 | Output Acc. |
| 6 | Add X | 200D | Add Y to Acc. |
| 7 | Jc Top | 7000 | If carry, start over (jump to “Top”). |
| 8 | Sta Z | 400F | Store Acc in Z variable. |
| 9 | Lda Y | 100E | Load Y into Acc. |
| 10 | Sta X | 400D | Store Acc into X. |
| 11 | Lda Z | 100F | Load Z into Acc. |
| 12 | Jmp Next | 6004 | Jump to “Next”. |
| 13 | X; 0 | 0000 |  |
| 14 | Y; 0 | 0000 |  |
| 15 | Z; 0 | 0000 |  |

Multiplication:

| # | Assembly | Code | Comments |
| --- | --- | --- | --- |
| 0 | Lda Y | 100E |  |
| 1 | Sta Count | 4010 | Store multiplier (Y) as Count. |
|  | Top: |  |  |
| 2 | Lda Count | 1010 |  |
| 3 | Sub One | 300D | For every addition, decrement Count. |
| 4 | Jc Next | 7008 | If Count > 0, go to addition. |
| 5 | Lda Result | 1011 |  |
| 6 | Out | E000 | Output result. |
| 7 | Hlt | F000 |  |
|  | Next: |  |  |
| 8 | Sta Count | 4010 | Store decremented Count value. |
| 9 | Lda Result | 1011 | Load previous sum. |
| 10 | Add X | 200F | Add multiplicand. |
| 11 | Sta Result | 4011 | Store sum as Result. |
| 12 | Jmp Top | 6002 | Do it all again. |
| 13 | One; 1 | 0001 |  |
| 14 | X; – | – | Multiplicand variable. |
| 15 | Y; – | – | Multiplier variable. |
| 16 | Count; 0 | 0000 |  |
| 17 | Result; 0 | 0000 | Product value. |

### 5.6 Sample Programs (continued)

Loop Counter:

| # | Assembly | Code | Comments |
| --- | --- | --- | --- |
|  | Top: |  |  |
| 0 | Add Inc | 2003 | Increment by “inc” value. |
| 1 | Out | E000 | Display number. |
| 2 | Jmp Top | 6000 | Do it again. |
| 3 | Inc; 1 | 0001 | Value to increment by. |

Floor Division with Modulo:

| # | Assembly | Code | Comments |
| --- | --- | --- | --- |
|  | Top: |  |  |
| 0 | Ldi 0 | 7000 |  |
| 1 | Or Num | A012 |  |
| 2 | Sta Res | 4016 | Res = Num. |
| 3 | Jz Result | 800E | If Num == 0, skip division. |
| 4 | Ldi 1 | 5001 |  |
| 5 | Add Cnt | 2014 | Increment Cnt. |
| 6 | Sta Cnt | 4014 |  |
| 7 | Lda Res | 1016 |  |
| 8 | Sub Den | 3013 |  |
| 9 | Jc Modulo | 700C | If Num < Den, store Mod and output Cnt. |
| 10 | Sta Res | 4016 | Store current value as Res. |
| 11 | Jmp Top | 6000 | Do it again. |
|  | Modulo: |  |  |
| 12 | Sta Mod | 4015 | Mod = Res. |
| 13 | Nop | 0000 | Optionally output modulo. |
|  | Result: |  |  |
| 14 | Lda Cnt | 1014 |  |
| 15 | Sta Res | 4016 | Res = Cnt. |
| 16 | Out | E000 | Output result. |
|  | End: |  |  |
| 17 | Hlt | F000 | End of program. |
| 18 | Num; – | – | Numerator/Dividend variable. |
| 19 | Den; – | – | Denominator/Divisor variable. |
| 20 | Cnt; 0 | 0000 |  |
| 21 | Mod; 0 | 0000 | Modulus value. |
| 22 | Res; 0 | 0000 | Quotient value. |

### 5.7 Concept of Operating Systems

Traditionally, a program is loaded into a computer’s memory and is executed until completed, at which point the computer halts. Only one program can be loaded into a computer at once and the computer must be reprogrammed before each use. Operating systems can be used in computers to elevate this limitation, allowing the user to select from hundreds of programs which have all been loaded into memory at once.

The simplest form of an operating system is a small program located at the beginning of the system’s memory which, referencing a user-defined value, can jump to the location of and execute many other programs. This can be represented in a flow diagram:

Retrieve number of program to jump to.

Is number zero?

Is number one?

Is number two?

Jump to and execute first program.

Jump to and execute second program.

Jump to and execute third program.

## Appendix 1 — Maintenance Information

Hardware Vulnerabilities	Page 25

Hardware Vulnerabilities, Maintenance Advice, Bad Circuitry, Bad Microcode, Parts List

Maintenance Advice	Page 25

Hardware

Bad Circuitry	Page 25

Hardware

Bad Microcode	Page 25

Hardware

## Appendix 2 — Technical Information

Instruction Reference Card	Page 35

Instruction Set Listing

Parts List	Page 25

Hardware

System Schematics	Page 36

Redacted

Data Sheets	Page 37

74LS161, 74LS189, 28C16, 74LS382

## Appendix 3 — Supplementary Information

Boolean Algebra (Logic)	Page 25

History, Logical Operations, Binary States, Truth Tables

Electronic Logic	Page 28

Logic Gates, Layers

Number Systems & Binary	Page 30

Overview, The Binary System, Conversion

Storing Negative Numbers	Page 34

Two’s Compliment